# SECTION 07 42 13

# WALL PANELS

# RAPIDWRAPCI SYSTEM

SPECIFIER NOTE: THESE SPECIFICATIONS WERE CURRENT AT THE TIME OF PUBLICATION BUT ARE SUBJECT TO CHANGE AT ANY TIME WITHOUT NOTICE.  PLEASE CONFIRM THE ACCURACY OF THESE SPECIFICATIONS WITH THE MANUFACTURER AND / OR DISTRIBUTOR PRIOR TO CONSTRUCTION OR INSTALLATION.

GUIDE SPECIFICATIONS:

THIS GUIDE SPECIFICATION IS WRITTEN ACCORDING TO THE CONSTRUCTION SPECIFICATIONS INSTITUTE (CSI) FORMATS, INCLUDING MASTERFORMAT, SECTIONFORMAT, AND PAGEFORMAT.

CAREFULLY REVIEW AND EDIT THIS SECTION TO MEET THE REQUIREMENTS OF THE PROJECT, LOCAL BUILDING CODE AND AUTHORITIES HAVING JURISDICTION. COORDINATE THIS SECTION WITH OTHER SPECIFICATION SECTIONS AND DRAWINGS.

DELETE ALL "SPECIFIER NOTES" WHEN EDITING THIS SECTION.

# GENERAL

## CONDITIONS AND REQUIREMENTS

### The General Conditions, Supplementary Conditions, and Division 01 – General Requirements apply.

### SECTION INCLUDES

    - Pressure Equalized rain screen system with continuous insulation (CI), designed to eliminate thermal bridging and improve energy efficiency.

## SUMMARY

    - The work of this section includes but is not limited to the design, fabricating, finishing, and erecting of the aluminum/metal composite panel system, specifically the RapidWrapCI system.

    - Provide an aluminum/metal composite panel system including aluminum/metal composite material panels, insulation, and support system mounting hardware, where shown on the drawings as specified, and as needed for proper installation. The RapidWrapCI system is designed to be an efficient and thermally continuous wall assembly with no thermal bridging, without sub-girts or insulation-penetrating structural stand-offs.

## RELATED SECTIONS

    - Section 07 42 13.23 – Metal Composite Material Wall Panels: For Vitrabond FR ACM panels.

    - Section 07 21 00 - Thermal Insulation: (such as polyisocyanurate or high-density mineral wool).

    - Section 07 05 43 - Cladding Support Systems.

### Section 05 40 00 - Cold-Formed Metal Framing.

### Section 07 60 00 - Flashing and Sheet Metal.

### Section 07 92 00 - Joint Sealants.

### Section 08 51 13 - Aluminum Windows.

### Section 08 81 00 - Glass Glazing.

### Section 08 44 00 - Curtain Wall and Glazed Assemblies.

## REFERENCES

    - American Society of Heating, Refrigerating and Air-Conditioning Engineers (ASHRAE):

- ASHRAE 90.1 – Standard for Energy Efficiency in Building Design.

This standard defines the energy efficiency criteria for building designs, including thermal performance and insulation, ensuring that the system provides continuous insulation without thermal bridging.

    - National Fire Protection Association (NFPA):

- NFPA 285 – Fire Resistance for Exterior Wall Assemblies.

This standard specifies the fire resistance requirements for exterior wall systems, ensuring materials meet the necessary fire safety performance criteria.

    - American Architectural Manufacturers Association (AAMA):

### AAMA 501.1 – Standard Test Method for Water Penetration of Windows, Curtain Walls, and Doors using Dynamic Pressure.

### AAMA 508 - Voluntary Test and Classification Method for Pressure Equalized Rain Screen Wall Cladding System.

### AAMA 2605 – Voluntary Specification, Performance Requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

#### American Society for Testing and Materials International (ASTM):

#### ASTM C297 – Standard Test Method for Flatwise Tensile Strength of Sandwich Constructions.

#### ASTM D1781 – Standard Test Method for Climbing Drum Peel for Adhesives

#### ASTM E84 – Standard Test Method for Surface Burning Characteristics of Building Materials.

#### ASTM E283 – Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

#### ASTM E331 – Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls, but Uniform Static Air Pressure Difference.

## SYSTEM DESCRIPTION

## System Type:

## Provide a RapidWrapCI fully-tested, NFPA 285 and ASHRAE 90.1 compliant continuous insulation (CI) wall cladding system that eliminates thermal bridging with no sub-girts or structural standoffs.

## Performance Requirements: Provide aluminum/composite panels, which have been manufactured, fabricated, and installed to withstand loads from deflection and thermal movement and to maintain performance criteria stated by the manufacturer without defects, damage, or failure.

## Deflection and thermal movements:

      - Normal Deflection: Provide exterior/interior wall cladding assemblies designed to withstand dead loads, wind loads, snow loads, and normal thermal movement without permanent deformation, while maintaining continuous insulation (CI) performance and eliminating thermal bridging.

## Dead load: As required by applicable building codes.

## Wind load: Uniform pressure (velocity pressure) of [insert design criteria] pounds per square foot, acting inward and outward.

## Anchor deflection at connection points of framing members to anchors.  Anchor deflection in any direction must not exceed 1/16 inches (1.6 millimeters).

## Thermal Movements: Allow for free horizontal and vertical movement, due to expansion and contraction of components.

## Buckling, opening of joints, undue stress on fasteners, failures of sealant, or any other detrimental effects of thermal movement will not be permitted.

## Fabrication, assembly, and erection procedures shall consider the ambient temperature at the time of the respective operation.

## SUBMITTALS

***********************************************************************************************************************
SPECIFICATION CONSTRUCTION NOTES: COORDINATE THE FOLLOWING WITH DIVISION 01 - GENERAL REQUIREMENTS AND, TYPICALLY, SECTION 01 33 00 - SUBMITTALS PROCEDURES. IT IS NOT NECESSARY TO CROSS-REFERENCE TO SECTION 01 33 00.

***********************************************************************************************************************

### Product Data:

#### Submit manufacturer's printed product literature, specifications, data sheet, and 3rd Party Listing.

#### Submit construction details, dimensions of individual components and profiles, and finishes for each type of metal wall and accessories.

### Shop Drawings: Submit fabrication and installation drawings indicating layout, details of edge conditions, joints, panel profiles and corners.  Submit shop drawings for product components including finish, color, texture, and anchorage and attachment system.  Distinguish between factory and field assembled work.

### Include details showing thickness and dimensions system parts, fastening and anchoring methods, locations of joints and the location and configuration of joints necessary to accommodate thermal movement.

### For installed products indicated to comply with design loads, include structural analysis data signed and sealed by the qualified Professional Engineer responsible for the preparation.

### Shop Drawings shall indicate surrounding construction as provided for the Project.

### Manufacturer’s Samples: Submit sample showing finish, color, and texture.

### Selected Samples: Manufacturer’s color chips or samples matching the colors, finishes, and patterns to meet the Architect’s color selection.

### Include separate sets of finish samples of aluminum substrate not less than 3 inches by 3 inches (76 millimeters by 76 millimeters), of each color and finish selected to obtain color approval.

### Quality assurance submittals:

### Include documentation proving current third-party code compliance certification from an ANSI accredited agency, demonstrating compliance of the system components with current applicable International Building Code (IBC) requirements.

### Include designed system installation instructions.

### Closeout submittals: Submit warranties specified elsewhere in this section.

## QUALITY ASSURANCE

### Qualifications:

### Installers: must be experienced in performing work of this section and have specialized in work similar to that required for this project.

### Fabricator: capable of providing field service representation during construction.

### Company with a minimum of five years of continuous experience fabricating aluminum panel systems of the type specified.

### List five other projects of similar size and scope including the approximate date of installation and name of Architect of each.

### Product Manufacturer: demonstrate a minimum of 5 years experience in manufacturing pre-finished solid aluminum, and rainscreen installation systems.

### Refer to Section 01 45 00 - Quality Control.

### Regulatory Requirements: Refer to Section 01 41 00 - Regulatory Requirements.

### Pre-Installation:  A pre-installation meeting shall take place to verify project requirements, substrate conditions, weather proofing, flashing, installation instructions, and warranty requirements.

### Field Quality Control: Comply with systems manufacturer’s recommendations and guidelines.

## DELIVERY, STORAGE AND HANDLING

### In accordance with Section 01 60 00 - Product Requirements.

### Comply with manufacturer’s/fabricator’s ordering instructions and lead times requirements to avoid construction delays.

### All materials under this section shall be delivered with the identification label intact, and be packaged, boxed, wrapped in manufacturer’s/fabricator’s original, unopened, undamaged containers, or be otherwise protected to assure complete protection from reasonable damage during shipment, storage, and handling.

### Factory apply PVC removable plastic film to protect finish of Vitrabond 4mm FR ACM/MCM. This film must be removed immediately after installation to avoid prolonged exposure to direct sunlight.

### Protect aluminum composite wall panels against transportation damage.  Provide marking/labeling to identify components consistent with drawings.

### Exercise care in unloading, storing, and installing panels to prevent bending, warping, twisting, and surface damage.

### Materials shall be stored in enclosed spaces, above ground, under protective covers.  Extreme care shall be taken to avoid contact with moisture, condensation, or materials which might cause staining, such as lime, cement, fresh concrete, or chemicals.

### Storage and Protection: Protect materials from exposure to harmful weather conditions and at temperature conditions recommended by the manufacturer/fabricator.

### Store panels, extrusions, and fasteners in a well-ventilated space out of direct sunlight.

### Protect panels and system components from moisture and condensation with tarpaulins or other suitably ventilated weather-tight coverings.

### Slope panels to ensure positive drainage and prevent water accumulation.

### Do not store panels where ambient temperatures can exceed 120 degrees Fahrenheit (49 degrees Celsius).

### Material must be acclimated prior to fabrication for a minimum of 24 hours at 60 degrees Fahrenheit (16 degrees Celsius).

### Avoid contact with other material that might cause staining, denting, scratching, or other surface damage.

### To prevent adhesive transfer to finish, exterior aluminum composite wall panels must not be stored for prolonged periods of time, in direct sunlight, or be subject to high heat prior to installation.

## PROJECT CONDITIONS

### Site measurements: Verify actual measurements and openings by site measurement before fabrication.  Show recorded measurements on shop drawings.  Coordinate site measurements and fabrication schedules with construction teams to avoid delays.

### System must have base extrusions that can be planed on the wall directly over the weatherproofing and be flashed during or prior to field measurements.  This allows protection of the weatherproofing during the panel fabrication process.  Measurements can be taken from these installed base extrusions to provide accurate panel dimensions. This allows the precise placement of centerline joints to adjacent material such as curtainwall mullions, concrete panel joints, and other design features.

## WARRANTY

### Project Warranty: Refer to Contract Documents for project warranty provisions.

### Manufacturer’s Warranty for Metal Composite Material (MCM): Submit, for Owner’s acceptance, MCM manufacturer’s standard warranty document executed by an authorized company official.

### Warranty shall be for finish and product performance.

### Panel System Fabricator’s Warranty: Submit, for Owner’s acceptance, Fabricator’s workmanship warranty document executed by an authorized company official.

# PRODUCTS

## MANUFACTURERS

### Specified Manufacturer: Rapidwrap by Fairview Architectural Products, 75 Peters Road, Bloomfield CT, 06002, 860-242-2711.  Email: helpdesk@fairview-na.com; Web: www.fairview-na.com.

************************************************************************************************************************
SPECIFIER NOTES: DELETE ONE OF THE FOLLOWING TWO PARAGRAPHS.

************************************************************************************************************************

### Substitutions: Not Permitted.

### Requests for substitutions will be considered in accordance with provisions specified in Section 01 62 00 – Product Options.

### CONTINUOUS INSULATION Pressure Equalized RAIN SCREEN WALL CLADDING SYSTEM

### Performance/Design Criteria: Aluminum Composite Pressure Equalized Rain Screen Wall Cladding System.

### The system is a true Continuous Insulation (CI) system that eliminates the need for additional components such as sub-girts and structural stand-offs, significantly reducing thermal bridging and improving energy efficiency.

### The system shall be a true free floating/non-sequential installed panel system.

### The system shall allow for any damaged or defective panel to be removed and replaced without removing adjacent panels.

### The system shall allow for panels and reveals to be replaced while not compromising the integrity of the system or results from the AAMA 508 testing.

### The system must comply with below reference standards:

### AAMA 508: Test Method and Specification for Pressure Equalized Rain Screen Wall Cladding System.

### AAMA 501: Dynamic water penetration of a maximum of 3.2 square feet at 12 pounds per square foot (0.30 square meters at 575 Pascals).

### ASTM E283: Air leakage between 0.11 and 0.13 cubic feet per minute (0.051 to 0.61 liters per second).

### ASTM E331: Static water penetration of a maximum of 3.2 square feet at 6.24 pounds per square foot (0.30 square meters at 299 Pascals).

### The system shall ensure continuous insulation coverage with no thermal breaks caused by structural mounting elements.

### The system must allow panels to be locked in place with a pull-out tested twist lock connection.

### The system shall allow for future replacement of panels without removing base panels.

### The overall system depth shall be determined by the combination of the panel system and the continuous insulation (CI) layer. The depth shall typically range between approximately 4-5/8 inches to 5-5/8 inches, depending on the specified continuous insulation thickness (3 to 4 inches).

### Attachment Extrusions:

### All system extrusions to be alloy 6063-T-6.

### Base and starter extrusions as required shall be planed plumb level and true per system requirements.

### Stiffeners:

### Alloy 6063-T-6.

### Stiffeners shall be spaced as required for flatness or per Professional Engineer calculations if required.

### Stiffeners shall be attached with mechanical fasteners to frame extrusions in conjunction with either structural silicone to inside of panel surface or VHB tape adhering stiffener to back of panel.

************************************************************************************************************************
SPECIFIER NOTES: DELETE ONE OF THE FOLLOWING TWO PARAGRAPHS BASED ON WHICH ARROWHEAD SYSTEM IS BEING SELECTED.

************************************************************************************************************************

ARROWHEAD LEAN:

### The panel fabrication system must have no exposed fasteners on the perimeter return legs.

### Panel depth will be 1/2 inch.

ARROWHEAD FLEX:

### The system shall have stiffeners mechanically fastened to panel frame extrusions for panel integrity.

### Anel depth will be 1inch.

## COMPOSITE METAL PANEL MATERIALS

### Basis of Design: Vitrabond FR 4mm Aluminum Composite Material

### Composite Metal Panels: Refer to all manufacturer’s product information and specifications.

#### Core:Fire-Resistant (FR) Core that meets fire safety and performance requirements.

#### Face Sheets: Aluminum alloy or another specified metal, minimum 0.020 inches (0.51 millimeters) thick for composite panels as follows:

#### Coil coated with minimum 70% polyvinylidene difluoride (PVDF) fluoropolymer paint finish that meets or exceeds the requirements of AAMA 2605 relevant to coil coating.

#### Thermally bonded in continuous process to core material without glues or adhesives.

#### Bond Integrity:  Bonds shall be tested for resistance to delamination as follows:

#### Bond strength: 1,000 pounds per square inch minimum, as required by ASTM C297.

#### Peel strength: 33.6 inch pounds per inch minimum, as required by ASTM D1781.

#### No change in bond performance after eight hours of submission in boiling water and after 21 days of immersion in water at 70 degrees Fahrenheit.

#### Fire Performance for 4 millimeters:

#### Flame spread: Less than 25 when tested as required by ASTM E84.

#### Smoke developed: Less than 450 when tested as required by ASTM E84.

#### Production Tolerances:

#### Width: Plus-minus 0.08 inches (2 millimeters).

#### Length: Plus-minus 0.08 inches (2 millimeters).

#### Thickness: (plus-minus 0.02 millimeter for 4 millimeter panels.

#### Bow: Maximum 0.8 percent length or width.

#### Squareness: Maximum 0.08 inches (2 millimeters).

#### Edges of sheet shall be square and trimmed with no displacement of aluminum sheet or protrusion of core material.

#### Material Thickness:4 millimeters for standard FR core as based on application, core composition, and local building codes.

## FABRICATION

### General: Shop shall fabricate to sized and joint configurations indicated on shop drawings and placement of base extrusions if wall and surrounding conditions differ from approved shop drawings.

### Form panel lines, breaks, and angles to be sharp and true with a surface that is free from warp or buckle.

#### Fabricate with sharply formed edges, with no displacement of aluminum sheet or protrusion of core.

### All panels shall be shop fabricated per the system fabrication manual.

## FINISHES

### Factory Finish: As required by AAMA 2605 if coil coated with Fluoropolymer resin-based coatings.

#### Color Selection: As noted on architectural plans. Note that any brand names on plans are for reference only for finish selection, and do not override this specification.

## ACCESSORIES

### General: Installer shall supply standard accessories including:

#### Fasteners.

#### Clips.

#### Anchorage devices.

#### Shims.

#### Backer rods.

#### Sealants.

## RELATED MATERIALS

### General: Refer to related sections regarding related materials including:

#### Cold-formed metal framing.

#### Flashings.

#### Trim.

#### Joint sealer.

#### Aluminum windows.

#### Glass and glazing.

#### Curtain walls.

# EXECUTION

## PANEL SYSTEM’S/MANUFACTURER’S INSTRUCTIONS

## Compliance: Comply with the panel system’s/manufacturer’s product data, including technical bulletins, fabrication manuals, installation instructions, and product(s) carton instructions.

## EXAMINATION

### Examine site conditions and verify substrate conditions previously installed under other sections are acceptable for product installation.

### Examine project conditions and completed Work and installations are acceptable for proceeding with panel system installation.

### Immediately correct all deficiencies and conditions which would cause improper execution of Work specified in this Section and subsequent Work.

### Proceeding with Work specified in this Section shall be interpreted to mean that all conditions were determined to be acceptable prior to start of Work.

## PREPARATION

### Substrate verification: Verify that the substrate is structurally sound, square, level, plumb, true, dimensionally accurate, and in accordance with approved shop drawings.

### Notification: Notify the Architect in writing of anything that could impact timely and accurate installation.

### Adjacent surfaces protection: Protect adjacent work areas and finish surfaces from damage during product installation.

## INSTALLATION

### Component parts that are observed to be defective in any way including warped, bowed, dented, abraded, and broken members, must not be installed.  Members or parts which have been damaged during installation or thereafter before substantial completion of the project shall be removed and replaced.

### No cutting, trimming, welding, or brazing of components which could in any way damage the finish, decrease the strength, result in visual imperfections, or failure in performance shall be executed during installation.  Components which require alteration shall be returned to the fabricator.  If necessary, replace with new components.

### Tolerances:

### All components shall be visually flat, level, true to line with uniform joints and reveals.

      - Maximum deviation for vertical members: 1/8 inches over 18 feet and 1/4 inches over 40 feet.

      - Maximum deviation for horizontal members: 1/8 inches over 30 feet.

      - All joints between panels shall be set at widths as shown on the drawings with a tolerance of 1/16 inches.

      - No two adjacent or perpendicular joints shall have a difference in width exceeding 1/8 inches.

### Anchorage of the cladding substructure to the building structure shall be approved by methods in accordance with the specific and approved shop and installation drawings.  Supporting brackets shall be designed to provide three-dimensional adjustments and accurate location of wall components.

### The full installation shall be in strict accordance with the fabricator’s instructions

## ADJUSTING

### Corrective Actions: Replace or repair Work to eliminate defects, deficiencies, and irregularities.

### Repair panels with minor damage so those repairs are not discernable at a distance of 120 inches or 10 feet (3.1 meters).

### Remove and replace panels damaged beyond repair according to Panel System’s replacement instructions.

### Remove protective film immediately after installation of panels to avoid prolonged exposure to sunlight.

### Remove damaged panels, protective file, and other debris attributable to Work of this section from the project Site.

## CLEANING

### Cleaning: Thoroughly clean the Work specified in this Section and adjoining surfaces].

#### Remove: Remove temporary covering and protection of adjacent work areas.  Remove construction debris from project site and legally dispose of debris.

#### Repair: Replace or repair damaged installed products.

#### Clean: Clean installed products in accordance with manufacturer’s instructions prior to Owner’s acceptance.  Remove extraneous matter and marks off the façade components in a manner which leaves the completed installation free of any streaking, spotting, or non-uniform appearance.

## PROTECTION

### Protection, General: In addition to general requirements specified in Section [01 50 00 – Temporary Facilities and Controls], [01 54 00 – Construction Aids], [Section 01 56 00 – Temporary Barriers and Enclosures], [Section 01 60 00 – Product Requirements], and [Section 01 77 00 – Closeout Procedures], comply also with the following requirements.

#### Protect Installed Panels: Protect the installed product’s finished surface from damage during construction.  Institute reasonable protective measures as required assuring that installed panels will not be damaged by Work.

# END OF SECTION